NORTH CAROLINA STATE UNIVERSITY

DEPARTMENT OF CHEMICAL AND BIOMOLECULAR

ENGINEERING

“DETERMINATION OF DIFFUSION COEFFICIENT, DENSITY AND VISCOSITY FROM METHANOL-WATER MIXTURE USING METHANOL WATER MIXTURE”.

DR. ERIK SANTISO

BY

TOLUWALASHE E. OMOTUNDE

DECEMBER 5, 2019

- Introduction

Molecular Dynamics (MD) simulations are some of the most useful tools for the investigating molecular fluids behavior under various conditions and more complex biological systems. Due to breakthroughs in computing power like parallel computing, quantum computing and so on, a very large range of the simulation experiments can be reached. These gives us a way to connect macroscopic properties by to the microscopic properties and also validate theoretical models to better understand things at the atomistic level [1, 2].

These computations typically take much more computer time than classical thermodynamic models (i.e. from a few hours to several weeks of computing time, depending on system size). In industrial applications molecular simulation helps in estimating and developing properties of unknown chemicals, extreme conditions of temperature or pressure, and toxic compounds. The first part of this article provides a brief description of Molecular dynamic simulation and how it can be analyzed for approving experimental data. Later in the report, different properties of the system will be calculated with molecular dynamic simulation. The key underlying issues for accuracy are adequate configurational sampling and the quality of the description of the intermolecular energetics. The estimated properties are then compared with available experimental data and theoretical models.

# Method

2.1- Why molecular dynamics method?

It is not possible to observe directly, phenomena that occur on the atomistic scale so we need a way to tell the story of the atoms as they move in space and time.  In molecular simulations, molecules are allowed to interact for a fixed period of time, giving a view of the dynamic "evolution" of the system. In the most common versions, the trajectories of atoms and molecules are determined by numerically solving Newton’s equation of motion for a system of interacting particles, where forces between the particles and their potential energies are often calculated using interatomic potentials or molecular mechanics force fields [3]. The equations solved are:

Fi = −miri 									(1)

Where:

Fi = − ∂U∂ri  									(2)

ri= d2ridt2  								(3)

Where Fi is force acting on particle I, ri is Acceleration of particle i and mi is mass of particle i. The force acting on each atom is obtained from a potential function U, such as that depicted below [4]:

U= Σkijb2rij- reqij2+Σanglekijθ2θij- θeqij2+Σdihedralskijϕ1+cosnϕ- ϕeq+                         Σi<j[Aijrij12- Bijrij6+qiqj4πε0rij]							(4)

Where the first three terms reflect intramolecular interactions between covalently bonded atoms and the last term describes the non-bonded interactions (van der Waals and electrostatic). The constants kb, kË, and kˆ   are force constants for the covalent bonds, bond angles, and dihedrals, respectively, rij are the distances between atoms i and j, Aij and Bij are the Lenard Jones parameters, and q is a partial charge [4]. An ensemble is typically used to estimate the properties of a real system. It is a very large number of systems, each constructed to be a replica on the macroscopic level. Ensemble averages of most average of dynamic property (A) can be gotten from:

<A> = ΣiAipi								(5)

Where Ai is the value of A in quantum state i, pi represents the probability of observing the i-th state, and the angled brackets denotes an ensemble average. The time averaged and the ensemble average are related by the Ergodic hypothesis which is as follows:

At→∞= <A>								(6)

2.2- Relationship to physical properties

Transport effects on microscopic scale usually have the highest effects on the properties of most chemical and biological systems and this is one of the major reasons why we perform simulations. Transport properties like diffusion coefficient, shear viscosity, partial density and pressure can be estimated directly from an MD simulation as it is not possible to observe them directly [5,6]. To get the self- diffusion coefficient DSα   of component α, you take the slope of the mean square displacement (MSD) at a long period of time using the Einstein equation. You can also calculate it by evaluating the integral of the velocity autocorrelation function using the Green-Kubo equations:

DαS= 12dNαlimt→∞< Σi=1Nα(riαt-riα0)2>				(7)

DαS= 1dNα∫0∞< Σi=1Nανiαtνiα0>dt					(8)

The shear viscosity and electrical conductivity can also be estimated using:

η= VVkBT∫0∞< σXYtσXY0>dt						(9)

ke= 13VkBT∫0∞< jetje0>dt						(10)

Where η is shear viscosity, keis the electrical conductivity, σXY is the pressure tensor of the components. kB is the Boltzman’s constant and T is the temperature of the system.

2.3- LAMMPS Software

LAMMPS is a classical molecular dynamics code with a focus on materials modeling. It's an acronym for Large-scale Atomic/Molecular Massively Parallel Simulator. LAMMPS has potentials for solid-state materials (metals, semiconductors) and soft matter (biomolecules, polymers) and coarse-grained or mesoscopic systems. It can be used to model atoms or, more generically, as a parallel particle simulator at the atomic, meso, or continuum scale. LAMMPS development began in the mid 1990s under a cooperative research & development agreement (CRADA) between two DOE labs (Sandia and LLNL) and 3 companies (Cray, Bristol Myers Squibb, and Dupont).

LAMMPS integrates Newton’s equations of motion for collections of atoms, molecules, or macroscopic particles that interact via short- or long-range forces with a variety of initial and/or boundary conditions. For computational efficiency LAMMPS uses neighbor lists to keep track of nearby particles. The lists are optimized for systems with particles that are repulsive at short distances, so that the local density of particles never becomes too large. On parallel machines, LAMMPS uses spatial-decomposition techniques to partition the simulation domain into small 3d sub-domains, one of which is assigned to each processor. LAMMPS molecular dynamics code contains several algorithms and force fields that have been developed over the years [8].

### Result and Discussion

#### 3.1-System definition

LAMMPS software is most efficient (in a parallel sense) for systems whose particles fill a 3d rectangular box with roughly uniform density. The system is defined by its size and shape, the number and types of molecules it contains, and the coordinates and velocities of all atoms. In this study MD simulation was carried out on a mixture of mixture of methanol and water to examine some physical and transport properties. This system consists of 600 molecules of water and 85 molecules of methanol. LAMMPS was used for the MD calculations. Simulation was performed using OPLS-UA [8] force field for 200 ps. Moltemplate was used to generate input files for the simulation which consisted of initial coordinates, bonds, angles, dihedrals and so on. The simulations were performed at four different temperatures 273, 283, 288, 293 k under atmospheric pressure using the microcanonical ensemble (NPE). Lenard-Jones parameters and partial charge for methanol molecule is seen in Table 1. Water molecules are modeled as TIP4P water model with their related parameters [11].

#### 3.2- Diffusion coefficient of the methanol in the mixture

As the first step, diffusion coefficient of methanol molecules in a mixture of methanol-water has been studied by MD method. VMD was used to visualize the motion of the molecules and calculate the mean diffusion coefficients [10]. Diffusion coefficients of the mixture was also calculated. The calculated results of the diffusion coefficient were compared to the values gotten from the Wilke- Chang equation [14]. Results are shown in Table 1 and Fig 3:

DAB= (117.3 × 10-18)(φMB)1/2TμVA0.6						(11)

Figure 1. The effect of temperature on diffusion coefficient.

The available experimental data for diffusion coefficient of methanol in water are shown in Table 2 [14]. The averaged error between MD results and experimental value is about 12%, while this is about 10% between experimental value and Eq.11.

| Temperature (K) | Diffusion coefficient by MD (cm2/s) | Diffusion coefficient by Eq. 11 (cm2/s) | Experimental value [11] |
| --- | --- | --- | --- |
| 273 | 1.22 * 10-5 | 0.98 * 10-5 | - |
| 283 | 1.62 * 10-5 | 1.18* 10-5 | - |
| 288 | 1.81*10-5 | 1.42* 10-5 | 1.56 * 10-5 |
| 293 | 2.01*10-5 | 1.60* 10-5 | 1.80 *10-5 |

Table 1. Calculated Diffusion Coefficient of Methanol in Water by MD method

3.3- Density

The density of the mixture was also calculated at different temperatures and points in the box using the density profile tool developed in VMD by Toni Giorgino and they are shown in Table 2 and Fig. 2 [19]. The corresponding results can be seen in Table 2 and Fig. 2. The error between experimental [14] and calculated results is less than 1% which shows a good agreement.

| Temperature (K) | Cal. Density (kg/m^3) | Exp. Density (kg/m^3) [11] | Error % |
| --- | --- | --- | --- |
| 273 | 934.10 | 928.70 | 0.0057 |
| 283 | 927.27 | 922.10 | 0.0056 |
| 288 | 915.26 | 918.50 | 0.0037 |
| 293 | 916.54 | 915.60 | 0.0008 |

Table 2. Calculated and experimental density in different temperatures

Figure 2. The effect of temperature on density for methanol-water mixture

3.4- Shear viscosity

Viscosities are calculated from the interactions of the molecules in microscopic scale.  The calculated viscosities were then compared with experimental data. From Table 4 and Fig. 3, it is seen that viscosity decreases by increasing temperature, the same as experimental values.

| Temperature | 273 K | 283 K | 288 K | 293 K |
| --- | --- | --- | --- | --- |
| MD Calculated value | 1.27 | 1.07 | 0.95 | 0.71 |
| Experimental value [13] | 1.06 | 0.9 | 0.85 | 0.74 |
| Error % | 17.8 | 14.3 | 8.6 | 4.23 |

Table 4. Average calculated and experimental viscosity of the methanol-water mixture

Figure 3. Viscosity of the methanol-water mixture at different temperatures

4. Conclusion

It has been shown using theoretical models and experimental data that MD simulations provide a very neat alternative to calculate density, shear viscosity, hydrogen bonding and diffusion coefficient for the mixture of methanol-water. LAMMPS software was used for this purpose. The simulation was performed up to 200 ps. The results show that the calculated diffusion coefficient is in good agreement with those obtained from the Wilke-Chang equation, both of which were approved by experimental data. The diffusion coefficient in all cases discussed increases linearly with temperature.

Density and Viscosity are also affected by temperature as it generally decreases with temperature with a decreasing deviation between MD and experimental data. MD is generally more reliable at higher temperatures. Overall, the results show the reliability of the MD method to calculate the physical properties of the systems. One can conclude that, this method may be used to determine the macroscopic properties of the systems, instead of practical methods in the laboratory.

References

- Monticella L., Kandasamy, S. K., Periole, X., Larson, R. G., “The MARTINI coarse grained force field: extension to proteins”., J. Chem.  Theory and Comput., 4, 819-834, (2008).

- Sadus., R. J., “Molecular simulation of fluid, theory, algorithms and object-orientation”., Elsevier Science B.V., (1999).

- Wikipedia contributors, "Molecular dynamics,"  Wikipedia, The Free Encyclopedia  https://en.wikipedia.org/w/index.php?title=Molecular_dynamics&oldid=926815689 (accessed December 5, 2019).

- Nobakht, A. Y., M. Shahsavan, and A. Paykani. "Numerical study of diodicity mechanism in different Tesla-type microvalves." Journal of applied research and technology 11.6 (2013): 876-885.

- Ungerer, P., Nieto-Draghi, C., Rousseau, B., Ahunbay, G., Lachet, V., “Molecular simulation of the thermophysical properties of fluids: From understanding toward quantitative predictions”, Journal of Molecular Liquids, 134, 71–89, (2007).

- Krishna, R., Wesselingh, J. A., “The Maxwell-Stefan approach to mass transfer”, Chem. Eng. Sci., 52, 861-911, (1997).

- Dubbeldam, D., Snurr, R. Q., “Recent developments in the molecular modeling of diffusion in nanoporous materials”, Molecular Simulation Journal, Vol. 33, (2007).

- W. L. Jorgensen, D. S. Maxwell, and J. Tirado-Rives. (1996) J. Am. Chem. Soc. 118 (45): 11225-11236. DOI

- Marcus D Hanwell, Donald E Curtis, David C Lonie, Tim Vandermeersch, Eva Zurek and Geoffrey R Hutchison; “Avogadro: An advanced semantic chemical editor, visualization, and analysis platform” Journal of Cheminformatics 2012, 4:17.

- Humphrey, W., Dalke, A. and Schulten, K., "VMD - Visual Molecular Dynamics", J. Molec. Graphics, 1996, vol. 14, pp. 33-38.

- Gunsteren W.F., Krugerg, V. and Berendsen, J.C., “Computer Simulation of molecular dynamics methodology, applied and perspectives in chemistry”, Angew Chem Int Ed Engl, 29, 992-1023, (1990).

- Avogadro: an open-source molecular builder and visualization tool. Version http://avogadro.cc/

- Treybal., R., Mass transfer phenomena, McGraw-Hill Company, third ed., (1890). [13] Perry, R. H., Perry’s chemical engineer’s handbook, McGraw-Hill Company, fifth ed., (1984).

- Perry, R. H., Perry’s chemical engineer’s handbook, McGraw-Hill Company, fifth ed., (1984).

- Nobakht, Ali Yousefzadi, et al. "Heat flow diversion in supported graphene nanomesh." Carbon (2017).

- Jorgensen, W. L., Madura, J. D. “Optimized Intermolecular Potential Functions for liquid Hydrocarbons”, Mol. Phys.  1985, 56, (1381).

- Zielkiewicz, J. “Structural properties of water: Comparison of the SPC, SPCE, TIP4P, and TIP5P models of           water”, J. Chem. Phys, 123, 104501, (2005).

- Yousefzadi Nobakht, Ali, and Seungha Shin. "Anisotropic control of thermal transport in graphene/Si

heterostructures." Journal of Applied Physics 120.22 (2016): 225111.

[19]   Giorgino T. Computing 1-D atomic densities in macromolecular simulations: The density profile tool for VMD. Computer Physics Communications. 2014 Jan;185(1):317–22. Available from doi:10.1016/j.cpc.2013.08.022 or preprint at arXiv:1308.5873.